iDVR4000

Digital Video Surveillance System
User Guide

Notices

★Please read the relevant prospectus before using the monitoring system; you could not use this goods until you know the function of everything part of it, it just ensured that there is a best function on it. Otherwise, everything is at your peril.

★ The host system power supply voltage of 220 volts +10%, if the voltage is instable or interference from other accidents, you choose high quality power (ie UPS) to ensure a good working system,

★On the system, host shall work at the room temperature (-10 +3 ℃ - 55 +2 ℃), a clean environment and keep it a good ventilation .

★ The system belongs to Our company’s proprietary software products,  please do not copy without permission, otherwise we will not be responsible for the stability of the machine , meanwhile, we will pursue copyright piracy through the legal channels .

Hardware configuration requirements

Server

CPU：More than Celeron 2G (Recommended 1.8G dual-core or more)

DDR：More than 256M（Recommended DDR512M）

Graphic Card：32M Support Overlay (Recommended 64M)

Network Card：10－100M

Display Resolution：1024*768(32Bit)

Client

CPU：More than P4 1.7G

DDR：DDR 256M

Graphic Card：64M

Network Card：10－100M

Display Resolution：1024*768(32Bit)

Features (H.264 hardware compression)

Operating Systems Windows XP/VISTA，compress mode H.264 .

Display Resolution 1024 x 768                                         Record Resolution 352 x 288（PAL standard）， 352 x 240（NTSC standard ）Or 320 x 240，640 x 480（D1）.

The system Maximum support 32 channel.

Support single-screen, 4 screen, 8 screen, 9 screen, 10 screen, 12 screen, 16 screen, 20 screen, 25 screen, 36 screen ,Support to display full-screen, support for auto-flip, can be multi-stage amplification.

Real-time multi-screen display / Record / Playback function, support multiple PTZ control, supports a wide range of alarm box.

Continuous video, timing video, dynamic video, event video, alarm video ,and so on.

Each camera can set the display name.

NTSC / PAL video display standard two options, 25 / s real-time preview, near real-time playback.

Regulatory function of video attributes to make the image more clearly.

Video multi-level image quality adjustable to change the storage file size, to facilitate the application of different occasions.

Mobile alarm video recording, video alarm sensitivity, pre-video time, adjustable delay time video to support the voices of alarm output.

Intelligent search, camera lens can play back all the time ( year / month / day / time period or long period of time for retrieval), automatic alarm video search can distinguish video events.

With single / multi-screen playback at the same time.

With local amplification , playback with frame-by-frame, fast playback, slow playback , the conventional player.

Image playback can be carried out to capture function, files backup, document editing functions.

Remote monitoring, remote PTZ control, color, video manual, shield and unshield , restart your computer, such as control.

Support the current maximum capacity of hard disk , also several large hard disk at the same time.

Hard disk management can use the hard disk of the largest and the smallest hard disk space set aside to enable the hard disk can be fully justified to use.

Automatically creates a log file (user logs, system logs, remote control log), easily search for record of the operation.

System shutdown can be set from time to time, time to resume function, to ensure high stability of the system.

Can be set to run automatically start monitoring software, power automatically shut down procedures, calls automatically restart, automatic restoration of the video.

The System has been rigorously tested, stable and reliable.

The System hardware configuration requires low, 32 channel video is displayed at the same time, CPU occupancy rate of below 50%.

The System can effectively prevented the system to collapse from repeating operation of the user monitoring system.

Contents

1、	Server Operating System	1

◆、Login System	2

1.1  Introduction of the main button interface	3

1.2	PTZ control button Introduction	5

1.3	Login System	6

1.4	Introduction the right mouse button function	7

◆、System Setup	8

2.1	Video card Setup	8

2.2	Video storage Setup	11

2.3 Motion detection Setup	12

2.4 PTZ Setup	13

2.5 Alarm Setup	14

2.6  Network Setup	16

2.7 User manage Setup	17

2.8  System settings and information	18

2、Client Operating System	20

◆、Login System	20

1.1  Introduction of the main button interface：	20

1.2	PTZ control button Introduction	22

◆、System Setup	23

2.1	Channel list Setup	23

2.2	Group Setup	25

2.3	System Setup	26

2.4	User Setup	27

3、Review V3.0	29

◆、File playback mode	33

◆、Cut Record File	36

◆、BackUp	38

4、technology parameter	40

- Server Operating System

Notices

Please read instruction book carefully before operating

Please do as instruction book strictly in the process of software,

please refer to instruction book to set Settings in the software, and pay attention to setting the course of the attention. The first use of the software system known as the initial user "admin", no password .users log in, please set the "admin" user's password.

settings for a particular set of channels are of immediate effect.

The first time you set the "video store" page of video data, such as storage disk, it is suggest that users restart the software after the completion of monitoring software, and do  not change the option of this setting page arbitrarily. This set of information related to the integrity of the video.

Must be properly set up alarm decoder and the port number (default decoder using COM1, alarm using COM2), suggest that users do not change.

When you add a user, please select the user's permissions carefully , add users to avoid undue authority.

In order to ensure long-term and stable system operation, please restart the software operation after finishing setting.

Important  statement :

If the operator does not read the instruction book carefully, there is no statement in accordance with the operation of the steps, leading to system instability, data loss, such as video anomalies by themselves , Our company does not undertake any responsibility .

◆、Login System

Monitoring system is installed, double-click on the Windows desktop "iDVR4000 Server" icon, enter the monitoring system boot interface, as shown in Figure 1:

(Fig.1)

In the start screen, user can see the steps to initialize the system, This feature can be found where the problem lies in the video card there is a problem immediately , after the completion of a successful initialization into the user interface (Figure 2).

(Fig. 2)

### 1.1  Introduction of the main button interface

Preview video window style function

TV-out function keys to choose

System log function keys

Shield/Unshield function keys

capture and browse the current channel selected images

Open/Close the current channel sound

Login/Logout System

Minimize / hide the main interface window

Exit System

To enter full-screen mode (press ESC key to switch)

Forward page switch monitor screen

Back  page switch monitor screen

Auto page switch monitor screen

Open PTZ Control Panel(see 1.1: PTZ control buttons introduced)

Open E-map

Enter Review

Enter System Setup

Tip: If you do not know the specific function of a button, mouse over the button to wait five seconds, the system will automatically prompt the button's function.

### PTZ control button Introduction

(Fig.3)

PTZ UP

PTZ DOWN

PTZ LEFT

PTZ RIGHT

PTZ AUTO

FOCUS FAR

FOCUS NEAR

IRIS OPEN

IRIS CLOSE

ZOOM WIDE

ZOOM TELE

Control PTZ or Speed Dome the first auxiliary switch / preset points

Control PTZ or Speed Dome the second auxiliary switch / preset points

Control PTZ or Speed Dome the third auxiliary switch / preset points

### Login System

Click Button Appear the following login dialog box(Fig.4)

(Fig.4)

User： admin

Password：   Default is empty  (After login in the system ,user can set the password)

Click the button "Login" to enter the system

※　Note: After the first login, please enter the user settings page (Fig.14) to set the "admin" user password.

### Introduction the right mouse button function

(Fig.5)

“Audio”   Start / stop monitoring the voice of the currently selected channel

“FullScreen”     		Fullscreen monitoring

“Chat”     			Internet customers with the text Contact / chat

“Hand record”  		Select the current channel to start / stop manual video

“All Hand record” 		Select the all channel to start / stop manual video

“E-map” 				Open the e-map

“Net Info”	         	Show remote access news

“Version Info”	     	Display software version information

“Process minimize”	Minimize the main window interface, which is equivalent to "hide" button

※  Note: PTZ control, capture and other functions for the purposes of each channel. For example: when the first channel into the operation, please click on the first channel, and then operate accordingly.

- ◆、System Setup

Click ButtonEnter Setup(Fig.6), First, show the video settings page

(Fig.6)

### Video card Setup

"Preview" section:

2.1.0 		"Channel" drop-down menu: used to select the channel to modify it.

2.1.1 	"Channel Name": select the channel to modify the current display name (up to 40 English characters and 20 Chinese characters)

2.1.2 	"Color Program" drop-down menu: the system can choose the default color of some commonly-used configuration, can also choose "Custom" and through the following brightness, contrast, color, sharpness, etc. to adjust the slider to meet your needs.

2.1.3 "Opening of the Preview"

The current channel is selected to show: playing：

“”:Can monitor this channel screen, Otherwise, this channel can not monitor screen

2.1.4 OSD Setup

"Show channel name":	””	Show the channel name

Otherwise,don’t show the channel name;

"Show time":	  			“” Display system’s date and time

Otherwise,don’t show the system’s date and time;

"OSD Translucent":      	“” Show the channel name and the date and time for semi-transparent

Otherwise, Show the name and date of access time for non-transparent

Note: The channel name and the date of the location can drag the mouse to the right screen name and the date of the time

2.1.5 LOGO Setup

“Browse”button: the location of the LOGO document, request picture size no larger than 128x64

“Opening LOGO”:  		“” Show LOGO,Otherwise don’t show the LOGO;

“LOGO Translucent”:“” Open semi-transparent icon

2.1.6  Button function description:

Button "default value" for the current channel selected for the system set up to restore the default state

Button "Copy to all" for the current channel parameters of all the settings are copied to all other channels

"Video" section:

2.1.7 "Channel" drop-down menu:  used to select the channel to modify it

2.1.8 “No video signal when the voice alarm”：

“” That the current channel sound without video signal

2.1.9 Video quality settings

Rate Control Model：For the current channel rate control approach;

Dynamic Stream: According to the actual video stream  record

Fixed Stream: Used arbitrarily set to the video stream

Mixed Stream: When the stream is greater than the actual settings of the system stream into the set-up code enforcement to reduce the bit-stream

Video Quality: Selected for the current channel set video video quality (divided into the best, better, in general, poor, worst, such as 5, the user can according to different environment, different set of needs)

Frame rate: Selected for the current channel set video frames (PAL :1-25 frame / sec, NTSC :1-30)

Space constraints：Selected for the current channel settings video file size of the maximum space

2.1.10 Coding type

QVGA The current video encoding settings for QVGA format, video resolution of 320 * 240;

CIF  The current video encoding settings for CIF format, video resolution of 352*288/240;

Half D1 The current video encoding settings for QVGA format, video resolution of 640*480/240, need to support by Video card;

※　Notes：QVGA is the default video format

2.1.11 stream type

Video stream：    Village Code does not include the audio stream

Audio and video stream：Village Code  include the audio stream

2.1.12 Video standard

PAL Set the current standard for the PAL video standard, CIF resolution of 352 * 288 (China region)

NTSC Set the current standard for the NTSC video standard, CIF resolution of 352*240

※　Note: Depending on the specific settings of the standard camera support

2.1.13 Button Help

Reference 2.1.6

### Video storage Setup

Click“Video settings” tags access to regular video and dynamic video settings (Fig.8)

(Fig.8)

“Plan video” Part：

2.2.1 "Channel" drop-down menu: Used to select the appropriate channel;

2.2.2 "Timing video " single-box：Click，plans to add video is timing video;

2.2.3 "Dynamic video" single-box：Click，plans to add video is Dynamic video;

Add /delete  plan video method：

First option is "Time video" or "Dynamic Video", and then choose the following time scale in a day within one week ,left-click represent adding the type of video, right-click represent the deletion of the type of video. Article time, each lattice represent half an hour.

2.2.4 Button function description:

Button“Select all”  Copy the current video mode to the access of seven days

Button “Clear all”  Clear the channel of the video mode all the way for seven days

Button “Copied to the daily”	 Copy the channel video mode on sunday to the other days

Button“Copied to all”	Copy the channel video mode every day to the other channels

“Select the video disk” Part：

2.2.5 “Select the video disk”

Check from the list of the disk used to store documents

2.2.6 “The length of the video file”

Set the length of the video file packing，Default value is 30 minutes

2.2.7 "Disk space reservations "

Settings for each video disk space reservations，Default value is 200MB.

2.2.8 "Opening of the cycle of video"

Check means that the opening, or not open.

### 2.3 Motion detection Setup

Click“Motion detection Setup" tags access to Motion detection Setup (Fig.10)

(Fig.10)

2.3.1 Channel： Used to select the channel to modify it

2.3.2 Motion detection sensitivity：The sensitivity of motion detection system (Range 1-9, 1 is the most sensitive，9 as the most insensitive, suggest that values is 2)

2.3.3 The length of pre-video: Motion is detected before time video (5-20s)

2.3.4 The length of time delay video：it will sustain video the time(5-20s)after the video system stop,

2.3.5 Motion triggered the voice：When motion is detected, it will sound.

### 2.4 PTZ Setup

Click “PTZ Setup" tags access to the PTZ control settings(Fig.11)

(Fig.11)

2.4.1 Channel：Used to select the channel to modify it

2.4.2 Type：  two types of decoder and fastball

2.4.3 Protocol type: Decoder decoding agreement

2.4.4 Use Port： Selected for the current channel set-top boxes connected to the serial number

2.4.5 Baud rate：Channel selected for the current settings of baud rate decoder

2.4.6 Start Address: Decoder address stream when setting the parameters, the value corresponding to the first channel decoder address stream.

2.4.7 Button Help：see 2.1.6

### 2.5 Alarm Setup

Click “Alarm Setup" tags access to the alarm parameters settings(Fig.12)

(Fig.12)

2.5.1 Alert Protocol: Alarm device communication protocol

2.5.2 User Port: Alarm equipment connected to the computer's serial number

※　Note: In general the ptz of serial cloud mirror 1, the alarm use the serial port 2 (not set to repeat)

2.5.2 Alarm channel: Select a channel alarm equipment

2.5.3 “Opening the channel alarm"： Whether the opening of the channel alarm

2.5.4 “Normally closed alarm input"：After check，Closed circuit alarm, said alarm without alarm, that alarm off, or vice versa.

2.5.5 “Alarm trigger sound"：Receipt of alarm signals, trigger the sound.

2.5.6 “Alarm pop-up E-map"：Alarm signal received, the pop-up E-map.

2.5.7  “Alarm Link video"：when this channel alarm，there will trigger the video card on which the channel record.

2.5.8 “Alarm Link output"：when this channel alarm，there will trigger the video card on which the channel alarm.

2.5.9 “Opening of the regular deployment": “” on behalf of arrange protection by the system automatically,otherwise,by the system manually operated.

### 2.6  Network Setup

Click “Network Setup" tags access to the Network parameters settings(Fig.13)

（Fig.13）

2.6.1 Channel：  Used to select the channel to modify it.

※　Note: in the video attribute settings, the selected channel if the current preview is not enabled, then the currently selected channel is also monitored through the network.

2.6.2 “IP address Verification”： After check, the IP address can be specified to allow the network to monitor, the IP has not been added and it will not be allowed to monitor.

2.6.3 “User Password Authentication”： After check, they can enter a user name and password to check the network connection

2.6.4 “Network port”： Specify the corresponding port.

2.6.5 "When the network connection shows" Network Connection Information "dialog box": When a network connection, whether show the "network connection information" dialog box

2.6.6 " Alarm upload ": Alarm signals received, whether upload the alarm signal.

### 2.7 User manage Setup

Click “User Setup" tags access to the user parameters settings(Fig.14)

(Fig.14)

2.7.1 User List：In this list all system users.

2.7.2 “User Permissions”： Choose to add the user permissions.

※　Note:	You must be careful to choose the user permissions when you add users , admin user can change password, but cannot delete and modify the permissions.

### 2.8  System settings and information

Click “System settings " tags access to the system settings(Fig.15)

(Fig.15)

“System”part：

2.8.1 “Shutdown Timing” When the system time set to the user settings of the regular time, the system will automatically implement timing

2.8.2 “Exit the program”:

Return desktop：   Exit from the system to the desktop;

Restart the computer：Restart the computer when you exit system;

Shut down the computer： Shut down the computer when you exit system;

2.8.3 “Switching time of automatic inspection”: Automatic inspection, the time length of the screen staying;

2.8.4 “Allow to use Task Manager”:

“” allow the user to use Task Manager

Otherwise,the user can’t use TaskManager

2.8.5 “Allow to use of WIN key (need restart system)”:

“” allow to use WIN key;

Otherwise,WIN keys are not allowed to use;

“System information”part：

This page shows the configuration of the computer and system uptime.

- 2、Client Operating System

- ◆、Login System

Double-click the "client" icon on the Windows desktop after booting, enter the client monitoring system, the main interface is as follows:

- 1.1  Introduction of the main button interface：

Preview video window style function

Connect to server function button

Packet connection / control, if the system is only one group, then automatically all connected;

capture and browse the current channel selected images

Open/Close the current channel sound

Login/Logout System

Minimize / hide the main interface window

Exit System

To enter full-screen mode (press ESC key to switch)

Forward page switch monitor screen

Back  page switch monitor screen

Auto page switch monitor screen

Open PTZ Control Panel(see 1.1: PTZ control buttons introduced)

Open E-map (Note: The current version of the client e-map function is unavailable)

Enter Review

Enter System Setup

### PTZ control button Introduction

(Fig.3)

PTZ UP

PTZ DOWN

PTZ LEFT

PTZ RIGHT

PTZ AUTO

FOCUS FAR

FOCUS NEAR

IRIS OPEN

IRIS CLOSE

ZOOM WIDE

ZOOM TELE

Control PTZ or Speed Dome the first auxiliary switch / preset points

Control PTZ or Speed Dome the second auxiliary switch / preset points

Control PTZ or Speed Dome the third auxiliary switch / preset points

1.3  Login System

Click the login button, and that will appear the following dialog box：

When the software used for the first time,you can only use the admin login user name(no password), the account with super user permissions, login below to access a list of monitoring points set up, monitor screen settings, system settings and user settings. as follows.

- ◆、System Setup

  - Channel list Setup  (As follows)：

The page set up to monitor the server and control point (camera)：

（1）Add Server：

In server Settings information bar,respectively, fill in IP Address、server name (optional) and choose  the service channel, click the add button. The list on the left add a server. Press the Apply button. add the server to complete.

（2）、Update server：

Select the server on the left list. In server Settings information bar , fill in IP Address、server name, click the update button. Click Apply and then. Update server completely.

（3）、Delete server：

Select the server on the left list. click the delete button. Click Apply and then. Delete server completely.

  - Group Setup(As follows)：

The page can generate packet monitor settings：

Settings,as follow these steps: In the packet drop-down list, select its packet number in the server list of the tree, select the server in the sub-list of the server monitoring points, select the monitoring point. Click on the "modify packet content" button to add the monitor screen; click the "Clear the group" button will clear all of the monitoring group settings. The completion of a group set up all the monitor screen, click on the application of the entry into force, to repeat the steps to set up other sub-groups.

  - System Setup(As follows)：

The page displays the computer system configuration and completes policy settings：

Automatic restart：Automatic restart your computer time，check effectively；

Automatic shutdown：Automatic shut down your computer time，check effectively；

Video storage disk set：Set the record documents and video capture documents and disk drives, the box will be automatically listed in the list of computer disk from the beginning of the hard drive C partition. Strongly recommended: Set the storage site after the E or the letter. If they choose to save in the E plates, while the video paper documents and drawings are located in E:\ REC_DATA and D:\ DVRCFG\V6\CLIENT\BMP directory,，this directory has two subdirectories, one is the file name .

Click on the application entry into force after the complete settings page.

  - User Setup (As follows)：

1）、Add User：

Click the Add User button. In the Add User dialog box fill in user name, password, confirm password. Select the appropriate permissions. Click Apply. Completion of the user to add.

2）、Delete User：

In the user list, select the user. Click to delete user button. Click apply. Completion of the user to delete.

3）、Modify the permissions：

In the user list, select the user. Select the appropriate permissions. Click apply. Completion of the user rights to amend.

4）、Change Password：

In the user list, select the user. Click the change password button. In the change password Change password dialog box. Determined. Click apply. Modify the user's password to complete.

The Permissions are as follows：

Record permissions：Allow to record；

Playback permissions：Allowed to play record files；

Capture：Allow to capture；

Messaging：Allows the user to send a message to the server；

Adjust color：Allows the user to remote adjust the color；

PTZ control：Allows the user to remote control the ptz；

Remote control：Allows the user to perform some special remote control commands（Control server to shutdown）；

Monitoring permissions：Allows to monitor the voices of the server；

Exit process：Allows the user to exit the process；

Set up a list of monitoring points：Allows to set up the monitoring points list, if the users log on without this Permissions, the setting page will not appear；

Monitor screen group setup：Allows to setup the monitor screen，if the users login in without this Permissions, the setting page will not appear;

System setup：Allows to setup system，if the users login in without this Permissions, the setting page will not appear；

User management setup：Allow for user settings ，if the users login in without this Permissions, the setting page will not appear.

- 3、Review V3.0

The review main interface is as follows:：

- The video documents show window：Execution of process, the default screen displays is 16, if necessary, could adjust for the 4 screen、 9 screen, 16 screen、25 screen or 36 screens. Enlarging a screen, the document will enter the single- screen mode. At any moment, there is a screen in the selected state, the program interface displays the local video ,file playback screen will display the information about the document. When a screen is selected, it's surrounded by a red box. Click the left mouse button with one of the display screen, the screen has been selected as the current playback screen;

- The number of video window：Adjust the display screen；

- Progress bar：The progress bar show the players document of the progress. The mouse can be dragged to adjust the location of file; The progress of the right figure 1/2/4/8/16 t or 1 / 2, 1 / 4, 1 / 8, 1 / 16 that reflect the current playback speed.

- Play：Normal speed to play the file；

- Pause：So that a file here suspended by clicking "Play" button to continue to play the file；

- Stop：The end of play and close the file；

- To file the first：So that the current document location of positioning at the first beginning, and documents in a suspended state,Clicke the "Play" button to continue to play；

- To file the end：So that the current document location of positioning at the end document, And documents in a suspended state, At this point, such as click on the "play" button, the document will be re-broadcast。Need to pay attention: because this function depends on the index of open files. the time and file size of the documents establish is proportional, so when the file is too large, the need to open the file after a certain period of time in order to use this function. Please note: In the "local playback" time, click on this button will open a document and from the beginning of a file to play；

- Slow：Slow down the playback speed of the document. Continue to click on the button, playback speed document Sustainable down. Playback speed in normal circumstances, click the button total of 4 times, the button becomes inactive；

- Fast：Speed up the playback speed of the document. Continue to click on the button, playback speed document Sustainable. Playback speed in normal circumstances, click the button total of 4 times, the button becomes inactive；

- Previous frame：The "screen of the current players" to play back one frame, if the screen is playing, and it will be automatically suspended, so that players documents remain in the back one frame of the screen;

- Latter frames：The "window of the current players" to forward one frame, if the window is in play, and will be automatically suspended, so that players documents remain in the forward one frame of the screen;

- Voice switch：Open or close the voice of the video file. Open the sound, only the "Current Play screen " will be broadcast to play  sound ；

- Multi-channel playback：Into the "local playback" mode, select the files for multi-channel playback, detailed operation look the "File player way";

- Backup：File backup feature, described in detail later；

- Save as：The video file playing in the " screen of the current players" is saved to the user specified directory. The user can modify the file name；

- Editing：Intercept part of the video file playing in the” screen of the current players" to save as the user specified directory. The operation see details of "document editing.

- Capture：Capture the current video frame play in the " screen of the current players", save as the D: \ DVRCFG \ V6 \ REVIEW \ BMP directory, file name similar to the format of "20090325-122341.BMP", were intercepted on behalf of "Year Month Day - Hour Minute Second";

- FullScreen：Click this button, the display screen will be displayed full-screen, it is necessary to withdraw from full screen, simply click the top left corner of the keyboard "Esc" key or one of the display screen on the right mouse button double-click;

- The network IP address input box：To conduct long-range playback, here you want to playback in the remote host's IP address；

- Local playback：Click this button to enter the "local playback" mode, which is the default playback mode procedures；

- Network playback：Enter the IP address in the network IP box, click on this button, the program will connect the specified host IP into the "network playback" mode. See details behind the operation "Player way"；

- Browse：Click this button, the pop-up dialog box as shown below：

You want to playback the file, click the "Open" button in the "Current Playlist screen " play the document playback；

- The date of selection box：Select the video date to play in this box；

- Channel selection box：In this box, select the channel to broadcast it；

- Video data list box：Select the date and the camera after show them in this box all eligible Documents, double-click the playback file in the "window of the current players" to play this document;

- Select the video file type：Box in the video showed a list of video files in different ways；

- The current time display： By the time of "hour: minutes: seconds" to show the "Current Playlist screen " broadcast live images of documents, that is the picture of the real time recording;

- Documents show the total time： By the time of "hour: minutes: seconds" to show the "Current Playlist screen " is the total playing time of a document;

- Has been playing time display： By the time of "hour: minutes: seconds" to show the "Current Playlist window" is playing ,the file has been broadcasting time；

- Documents show the current players：The file name of playing in the "current Playlist window"；

- Exit：Click this button，exit the playback process.

- ◆、File playback mode

Currently offers four ways to play：

- Local Play：

This is the default mode of playing. After entering the procedure, we can see that in the upper right corner of the procedure, there is a marker of "date" drop-down box, a marker of "channel" of the drop-down box, the following box and a file list. Proceedings will automatically search all local hard disk video data. In the "Date" drop-down box can be seen that all the video files are added to the date of the drop-down box; In the "Channel" drop-down box, you can choose which one would like to broadcast channel; In the "Date" and " channel" are a good choice, if the designated channel in the specified date, there are video files in the document list, you can see video files the channel all day. Double-click the file list with the mouse any file name box, the document will be in the " screen of the current players" to play; also can click any of the file list file name box, if this " screen of the current players" did not play any document, Then click on "Play" button, then click the file previously will be broadcast；

- Network Play：

This is a broadcast network files on other machines to provide a function. The upper right corner in the process, there is a "network IP address input box", enter the target machine IP, and then use the mouse to click on the "Network" button, the program will search all machines to allow access to the machine's hard disk, if finding video files, it will fill in the date drop-down box of their "date", and fill in the "file list box" of the appropriate file at the same time. During a time when the network broadcast, I would like to return to the "list of local broadcast" method, simply use the mouse to click on the "local" button, then return to the "list of local players" method.

Note: The network would broadcast on the target machine , the video file is set to share the letter (of course, can set up a number of privileges such as read-only, only certain account access). Moreover, the letter must be shared and not shared storage of video files. When the correct input IP address and click on the "Network" button , it may take some time to retrieve documents, depending on network speed and size of video data.

- The local browser to play：

Procedures for the upper-right corner click the "Browse" button, it will pop up a "Open File" dialog box, you can choose any one of the procedures to support the video file, click "OK", the document will be broadcast；

- The local row player：

Click on the "multi-channel continuous play" button to pop up a dialog box. As follows：

In the upper right corner of the dialog box, you can choose to play a normal file or backup file;

In the upper left corner of the dialog box, you can choose the approach at the same time intervals, from 1-16;

According to " intervals approach at the same time " is different, in the following  region "Please select the lens correspondingly with play region corresponding", the number of drop-down box will be dynamically adjusted, the number of " intervals approach at the same time " maintain unanimously. Take a look at this intervals at the same time following 16 plans:

In the region of "Please select the lens correspondingly with play region corresponding" , "the first screen" said the show screen of the first broadcast screen, the "second screen " that is the show screen of the second video display screen, as analogy. Taking into account the time of playback, consuming system resources, we only provide continuous playback of up to 16 at the same time, in actual operation, the machine choose the greatest playback approach according to the different configuration options at the same time. Recommended intervals at the same time do not more than 4-way approach.

Please note: the "first road" and "Channel One" are two different concepts. "The first way," referring to the program used to display video images of the first document path screen, and "Channel One" refers to the video file, the video card on the first channel, when playing video files, it can also be saw the video in the "No. 1 channel" message. And so on, "second way" and "second channel" concept is also different.

The user to choose the "Playback at the same time approach", we will show the default configuration for each screen to play the way the channel name, the "first screen" corresponding to "Channel 1", "second way" corresponding "Channel 2", the user can configured according to their own needs, by clicking with the mouse each way the drop-down box below, from the drop-down box select the appropriate channel.

End of channel choices, and then select the file date and time to start playing, click "OK", you can watch the playback of images.

In the designated "start date", the program will automatically search for video files on your hard disk, if a day corresponding to the video file (based on the player above to choose a "normal file" or "backup file"), then the date of this day will be added to "start date" drop-down box, so in the "Start Date" the date of the drop-down box are at least one channel of video files.

In some cases, one day may have a video file, but with regard to the choice of the above, the document did not play out. At this time, make sure you specify the channel in the days have video, at the same time, to confirm whether or not you start later than the time set. Because the procedures will determine that whether a file should play is to determine the beginning of the document recording time after the specified date. For example, we have only one day have a video on 1:00 p.m, but in the choice of "continuous multi-channel playback," when the set start time is 2:00 p.m., it will not play, because the video file in the specified time before the playback time.

Recommendation: Select the start time, if you do not know what the time of the video, just "hour" and " minute" are set to 0, so that from 0:00 to start the day's search, all in the same day after 0:00 video will be broadcast。

- ◆、Cut Record File

Player in the document, click the "edit" button, the following pop-up dialog box：

In the "scope" the region, it can be seen in frames and in seconds the length of the document. From the above picture can be seen，a total of 8100 frame of the document, or there is a length of 328 seconds.

In the "starting position" and "end position" of the right are two different input box ,for the left side of the Interception of user input to the start position and end position (end position must be greater than the initial position), the right is calculated after the procedure, the documents come to the interception of the end of the initial location and location. In practice, the user enters the start position and end position and the procedure to calculate the start position and end position may be about 1 ~ 2 seconds error. When the interception in the document in order to calculate the location of the procedure shall prevail.

Click the" choose the directory" button  on the  right of "Save file name" , you can open a "Save As" dialog box for users to choose the path of files. In the "Save As" dialog box modify the file name of the storage operation is invalid, because the process will automatically be editing the document in accordance with the time and clip recording to calculate the starting point of the document after editing the file name, file name and file to ensure that starting Time line before.

"Save the file name" the interception of the following documents is to show the path and file name.

As a result of the above options, click on "View" button, the program automatically calculates the document on the interception of the initial position and end position, and calculate the interception of the file name. As follows：

The "X" in the file name indicates that the document is a paper clip.

After completion of the operation, click the "Save the file", will be editing the document, the document will not be editing the paper clip to cause any damage, it is only from the paper clip to copy a clip of the document saved to the clip;

- ◆、BackUp

Click "Backup" button, the dialog box from the following：

First of all, choose to back up the channel, the choice of a good after the "back-up start date" and "end backup date" drop-down box, the program will automatically fill in the channel video files stored on the local machine all of the date, select the "backup start date "and" end backup date "and, if necessary, also need to enter" backup start time "and" back up the end of time. "  the last option choose a disk to backup.

After selection, click the "Cancel the backup" to cancel this operation, you can click on the "Start Backup" to back up. Click the "Start Backup", the program starts to backup the file size of the total space, and then back up the purpose of verifying whether there is sufficient disk space for storage, if space is not enough will be prompted. If space verification is passed, please wait for some specified period of time, recording procedures will have to back up the files to a specific drive letter, and in the letter to create REC_DATA_B directory, then copy files to this directory. In this directory, and then the document recording time, channel to create a different subdirectory.

File backup process is as follows:

If the backup time and found the same file name of the file already exists, that is shown on the E: \ REC_DATA_B \ 2003-03-12 \ 12_00 \ CHL01 \ 12-35-56N.H64 already exists, will pop up the following Tips:

At this point, click "Yes", it will cover pre-existing swap files, behind the backup to continue. If the next time the same file name found in the file exists, the prompt will pop up; point of "no" does not cover the back of the backup to continue. If the next time the same file name found in the file exists, the prompt will pop up; click the "Cancel", it will cover pre-existing swap file, and the next, and then found the same file name of the file exists, drop it on the cover will not be prompted.

Backup is completed, the following dialog box will appear:

If the specified channel did not file at the designated time period, it will appear the following tips:

Time period set by the user does not record video, adjust the backup time period.

Tip: We have a file to determine whether a specified period of time to file, whichever is the beginning of recorded time, such as: one at 13:00 to start recording a document, ending at 14:00,  if the user specified  12:00 to 13:30 backup the video, the file will be backed up, because the documents were recorded in the designated period of time; if the user specified 13:30 to 14:30 to backup the video, the file will not be backup, since the beginning of recorded documents at the designated time outside.

- 4、technology parameter

| Model specifications |  |
| --- | --- |
| Platform | WINDOWS XP/VISTA Operating system |
| Video input | Multi-channel PAL / NTSC composite video input |
| Video output | Support the secondary display device |
| Audio input | Multi-channel audio input |
| Audio output | Through the sound card, speakers in real-time playback, real-time sound monitor site |
| Alarm input | 8 / 16 (normally open, normally closed optional) alarm input, can be expanded to a 32-way |
| Alarm output | 8 / 16 (normally open, normally closed optional) alarm output, can be expanded to a 32-way |
| Communication Interface | Standard RJ-45 Network Interface |
| Image Resolution | 352*288dpi PAL standard、352*240dpi NTSC standard |
| Display Mode | Real-time display, single-screen /4 screen / 6 screen /8 screen / 9 screen / 10 screen / 12 screen / 16 screen / 20 screen / 25 screen / 36 screen / full-screen display, lock, security function, the screen before and after the switching function |
| Screen information | The camera name, date, time, video, and alarm status, confidential cover regional |
| Time Show | Displays the current time, location, X (0-180), Y (0-270) adjustable, stored in the video frame, time display adjustable brightness, adjustable transparency |
| Lens Name | Shows that the camera name, location, X (0-280), Y (0-270) adjustable video frame stored in the camera name display brightness adjustable, adjustable transparency |
| Shield function | Screen located 128 *64 and 64 * 32 the size of the shielding region, the location X (0-220), Y (0-220) adjustable, stored in the video screen, the screen background color adjustable |
| Record Mode | Regular video、manual video、alarm video、 dynamic video、 boot / auto-start video |
| Record Quality | The quality of each and every way the best, better, in general, poor, worst arbitrary adjustable |
| Compression | H.264 compression |
| Video Media | IDE / SATA hard drive capacity limitation |
| Search Playback | Retrieved by date time and list of files, multi-screen broadcast at the same time, single-frame playback, fast playback, slow playback, continuous playback on time, single-screen zoom, partial zoom and other functions, save a single picture, remote playback |
| Backup | Can backup to another hard disk or burn, print image data |
| PTZ Control | Terminal through the RS-232 to 485 communications, connect the camera to control PTZ decoder to achieve cloud mirror control |
| Dynamic Perception | Sensing region 99 can be set, 9 adjustable sensitivity, adjustable video duration, to support pre-video feature |
| Alarm mode | Video loss alarm, alarm signal from the alarm linked to a number of cameras, alarm time continuous video is adjustable linkage alarm output and alarm output can be more than a row at the same time, the output time adjustable |
| E-map | Automatic pop-up E-map when alarm |
| Remote network | PSTN、ISDN、ADSL、DDN、INTENET、Fiber |
| Network functions | Real-time monitoring, remote playback, remote recording, remote control cloud mirror, remote video attribute adjustment |
| Remote Alarm | Voice prompts, images text prompts |
| Remote Control | Video remote control, monitoring, PTZ and so on, remote control of the network server shutdown or restart |
| System Protection | Multi-level password management, automatic power shutdown, not the loss of video information, call the restoration of the video starts automatically, and turn off the automatic restart procedure to realize unmanned |
| Seamless connectivity | Replace video file to use of seamless connectivity to ensure that video is not dropped frames. |