- Any line starting with "*" is a comment.

- "unit I" is imperial units (all numbers in the file are in inches).
"unit M" may be metric (all numbers in the file are in metric, presumably millimeters).
Any other letter than "I" will be considered metric by Frontend*.

- "wood #.####" is the dieboard thickness (used for notch heights).

- "total #.####" is the total rule in the file (unused by Frontend).

- "id text #,#":
The numbers are the X and Y coordinates for displaying the piece number (supported in
		Frontend version 2.32 and up).

- "rule #,#":
The first number is the rule piece identification number (1 to 32768, each piece must be unique).
The second number is the quantity for that piece.

- "note" can be followed by notes for that piece (unused by Frontend).

- "lipnone" means straight cut (for knife/cut rule).
"lipnone" also means radius cut (for crease rule).
"lipbutt" means straight cut (for crease rule).
"lipleft" means miter cut at the start of the piece.
"lipright" means miter cut at the end of the piece.
"liptaper" means use a tapered straight cut tool, usually for butting curved rule to the side of 
		straight rule.
"lipnotch" means use a  trimknife tool (one side of the tool cuts a notch off the top of the rule,
		the other side cuts a miter); only used as the ending cut, never the starting.

The starting cut of the piece has no number directly after the cut type.
The ending cut is followed by the piece length, i.e. "lipnone #.####".

New format for “lipleft”, “lipright”, and “lipnotch” (supported in Frontend version 2.15 and up):
	The starting cut for "lipleft" is followed by "onto #.####", i.e. "lipleft onto 0.9900".
	The ending cut for "lipright" is followed by the piece length and also by "onto #.####",
		i.e. "lipright 9.8765 onto 0.9900".
	In both cases, the number after "onto" is the height of the rule being mitered onto, and this 		height is used to cut the miter tip.  For the trimknife tool, the “onto” value is only 			applied to the miter half of the tool.

- "start #.####,#.####,#.####":
The first and second numbers are the X and Y coordinates of the starting end of the piece in the 		total die layout.
The third number is the angle of the piece from the start coordinates.

- "type #.####,#.####,TYPE1,TYPE2,"RULENAME"[,ALIGNMENT]":
The first number is the rule height.
The second number is the rule width.
TYPE1 is either "cut" or "crease" or "perf".
TYPE2 is either "flat" or "rotary".
RULENAME is a descriptive name entered by the user (must be in quotes, e.g.
		e.g. type 0.9700,0.0560,cut,rotary,"970 4pt 66x90 Cut"
		and must not contain quotation marks (“) within the name).
ALIGNMENT is optional; the words “align” or “noalign” may be added to the end of the line.
		If neither is added, “align” will be assumed.  The “align” setting causes the first 			bridge of each curved piece made with notched rule to be aligned to a tooth.
		If “noalign” is set, the notches will not b aligned and the bridges will be placed 			wherever they happen to fall.

- "notch #.####,#.####":
The first number is the notch position from the start of the piece.
The second number is the notch width.

- "bend #.####,#.####":
The first number is the bend angle (+ counter-clockwise, - clockwise).
The second number is the bend position from the start of the piece.

- "curve #.####,#.####,#.####":
The first number is the radius of the arc.
The second number is the arc start position from the start of the piece.
The third number is the included angle of the arc.

An arc that curves clockwise can be represented using EITHER a negative radius OR a negative 		included angle.  Frontend supports both formats because a negative included angle is 		preferable, although Cimex outputs a negative radius instead.

- "endrule" ends every piece.

- “start of design” marks the start of the design section (for displaying the non-rule die layout).

- “from #.####,#.####” (design section):
The numbers are the X and Y coordinates of the start of the line or arc.

- “begin #.####” (design section):
The number is the beginning angle of the line/arc from the start coordinates.

- “end #.####” (design section):
The number is the ending angle of the arc from the start coordinates (not used for lines).

- “to #.####,#.####” (design section):
The numbers are the X and Y coordinates of the end of the line or arc.

- “radius #.####” (design section):
The number is the radius of the arc (not used for lines).  The radius will be positive for an arc
		that curves counter-clockwise, and negative for clockwise.

- “end of design” marks the end of the design section.

Written by:  Erika Baril, Data Technology, Inc.

Last modified:  9/13/2000