��������������������������������������������������������������������������Ŀ

�         TE2008 ODI DRIVER INSTALLATION FOR NOVELL ADVANCED NETWARE       �

����������������������������������������������������������������������������

1.0  Novell NetWare ODI Driver Installation

�������������������������������������������������

Follow the steps outlined below to create a driver diskette for use

with Novell Advanced NetWare.

1.1  Preparation

�������������������������������������������������

Before you start with the workstation installation process make sure

you have the following items on hand:

1) TE2008 adapter card

2) TE2008 driver diskette

3) Novell WSGEN diskette.

Create a BOOT diskette (this diskette may be self-booting, or it may

simply be a DOS-formatted blank diskette).

1.2  How To Install...

�������������������������������������������������

1) Copy the following five files into your BOOT disk.

a.  LSL.COM      �� supplied by Novell in the WSGEN diskette.

(b.) RPLODI.COM   �� Optinal, it's necessary for remote booting.

supplied by Novell.

c.  2008ODI.COM  �� 2008ODI Driver in the \ODI\DOSODI subdirectory

of the TE2008 adapter diskette.

d.  IPXODI.COM   �� supplied by Novell in the WSGEN diskette.

e.  NETx.COM     �� supplied by Novell in the WSGEN diskette.

� or �

EMSNETx.EXE  �� (NETx program with EMS support)

� or �

XMSNETx.EXE  �� (NETx program with XMS support)

f.  NET.CFG      �� Optional configuration file supplied by the

subdirectory of the TE2008 adapter diskette.

Note:   TE2008 default frame type is Ethernet_802.2,

but the default frame type is Ethernet_802.3

for Netware 3.X, Ethernet_802.2 for Netware 4.0.

1.  Copy NET.CFG\802.3 to NET.CFG for Netware 3.X

2.  Copy NET.CFG\802.2 to NET.CFG for Netware 4.0

2) After booting the workstation, type the following commands on

the console:

LSL     <Enter>

2008ODI <Enter>

IPXODI  <Enter>

NETx    <Enter>

You may wish to write all commands into a batch file and execute the

batch file instead of typing all the commands each time the workstation

needs to be booted up.

1.3  NET.CFG for TE2008

������������������������������

The NET.CFG file is used to provide the software configuration to the

ODI driver on the NetWare workstation.  Typically, all of the required

operating parameters will have been preset by TE2008 utility and matched

with file setting.

You can edit the NET.CFG file that is on the current directory in

order to change, add or delete any desired options by way of a plain-

ASCII text editor (DOS 5.0's EDIT and EDLIN commands are good examples

of plain-ASCII text editors).

�������������������������������������������������������������Ŀ

� IMPORTANT: There can only be ONE file named NET.CFG in the  �

� same floppy disk or pathed directory.  If the driver finds  �

� more than one NET.CFG file, it will become confused and may �

� not work reliably.  MAKE SURE THERE IS ONLY *ONE* NET.CFG!  �

���������������������������������������������������������������

PORT   : Indicates the base I/O address for the TE2008 network card.

type  : [hex]

range : [0x200,~,0x300,~,0x380] for TE2008 card

INT    : Indicates the  hardware interrupt  being used by the

network card.

type  : [decimal]

range : [2,3,4,5,10,11,12,15] for TE2008 card

FRAME  : Indicates the link-level frame type.

Note:  The default frame type is Ethernet_802.3

for Netware 3.X.

The default frame type is Ethernet_802.2

for Netware 4.0.

1.3.1 Example NET.CFG

�������������������������������������������������

Following is an example fragment from a NET.CFG file.  This fragment

lists the TE2008 options described above.  If this is the first time you

will be creating a NET.CFG file, note the layout of the NET.CFG fragment

below.  Spacing in the layout of the NET.CFG is important for reliable

operation of the adapter and network connection.

��> Link Driver 2008ODI

�����> PORT 300

�����> INT 2

�����> FRAME ETHERNET_802.3

�

��� The line "Link Driver 2008ODI" must not be indented at all.

2.0  Novell NetWare ODI Server Driver Installation

��������������������������������������������������

Installing the Ethernet drivers into a server computer is outlined below.

2.1  Preparation

��������������������������������������������������

Before you start with the driver installation process to the server,

make sure you have the following on hand:

1) TE2008 Ethernet adapter card

2) TE2008 Driver diskette

2.2  How To Install...

��������������������������������������������������

Installation of the Ethernet driver to a server is broken down into two

parts: Hardware Installation (changes to existing setting in the adapter)

and Software Installation (copying the appropriate driver to the server

and loading).

2.2.1 Hardware Install

��������������������������������������������������

Installing the TE2008 into a server is the same with the installation

of workstation. You just setup the hardware configuration by software,

and record the final setting values.

2.2.2 Software Install

��������������������������������������������������

1) Login to the server as the "Supervisor" user.

2) Copy the server driver supplied in the TE2008 Software diskette's

\ODI\SRVODI\NET3.X subdirectory into the server's SYS:SYSTEM

directory for Netware 3.X  or copy the \ODI\SRVODI\NET4.x

subdirectory into the server's SYS:SYSTEM directory for Netware 4.0.

3) Use the LOAD command from the System Console to load the 2008ODI.LAN

file into the server's memory.  Normally, the driver would load from

the SYS:SYSTEM directory of NetWare partition, but it could optionally

be loaded from a floppy disk or a directory on a DOS partition of the

server's hard disk instead.

A typical command line to load the server driver is:

LOAD 2008ODI.LAN PORT=300, INT=3

The LOAD line options may be entered in any sequence and the commas

in between them are optional, as are most of the options themselves.

However, the PORT and INT options MUST be included and they MUST equal

to the hardware settings.

Following are the parameters that may be included in the LOAD line:

PORT : The Base I/O Address at which the TE2008 adapter hardware has

been set. A drive denominator may be added to the port address

in order to specify a driver location other than the SYS:SYSTEM

directory of the NetWare Partition.  For example:

... PORT=300 ... <���� Load driver from SYS:SYSTEM (default)

... PORT=300:A ... <�� Load driver from floppy disk A:

INT  : This is the interrupt level (IRQ) that the adapter has been

configured to use.

4) You may optionally wish to put the LOAD line in the STARTUP.NCF or

AUTOEXEC.NCF auto-load driver files so that the LOAD statement will

take effect when the server system is initialized.

Closing Notes

��������������������������������������������������

Appendix "E" of the Novell NetWare 386 Installation Guide provides more

information about the usage of NET.CFG files.

=================== FILE ODI\INSTALL.DOC -- END OF FILE ====================