August 1992

SCO XENIX V2.3 Driver Installation

for

DE-400 EISA Bus Master Ethernet Card

==================================================

The following files will be used in this installation:

Files           Contents

-----------     -----------------------------------------------------

de400z.o        DE-400 Driver Program

dlinkiv.S

dlk4

rmdlk

tcp

strcf

1. Login as the root user.

2. Make a new directory /usr/sys/inet/dlink

# mkdir  /usr/sys/inet/dlink

3. Installing TCP/IP software through the custom utility.

# custom

Select install

Select A New Product

Select entire to install all of the packages

4. Rename /usr/lib/mkdev/tcp to /usr/lib/mkdev/tcp.save if you want to keep

original /usr/lib/mkdev/tcp file. (it will be modified by next step)

5. Installation diskette in dos format

Put your D-Link Installation Diskette to floppy A and use doscp command

to copy D-Link SCO Xenix Driver to following directories.

# doscp A:/scoxenix/readme.doc  /usr/sys/inet/dlink/readme.doc

# doscp A:/scoxenix/de400z.o  /usr/sys/inet/dlink/de400z.o

# doscp A:/scoxenix/dlinkiv.s  /usr/sys/inet/dlink/dlinkiv.S

# doscp A:/scoxenix/dlk4  /usr/lib/mkdev/dlk4

# doscp A:/scoxenix/rmdlk  /usr/lib/mkdev/rmdlk

# doscp A:/scoxenix/tcp  /usr/lib/mkdev/tcp

# doscp A:/scoxenix/strcf  /etc/strcf

# chmod +x /usr/lib/mkdev/dlk4

# chmod +x /usr/lib/mkdev/rmdlk

# chmod +x /usr/lib/mkdev/tcp

** Note: The extended portion of file dlinkiv.S should be upper case 'S'.

6. Installing D-Link SCO XENIX Driver

# mkdev dlk4

Do you wish to install or delete the dlk4 driver? type i to install

A display of information about driver.            type y to continue

Select interrupt vector number                    value

Which slot ( in hexdecimal )?                     input 1 to F

Do you want relink the kernel?                    type y to relink

Do you want this kernel to boot by default?       type y to default

7. Configuring Network interface

# mkdev tcp

HOST name                                       enter name

DOMAIN name                                     enter key

IP address                                      n.n.n.n

broadcast address of all 0's?                   n

Would you like TCP to start automatically?      [y/n]

8. Reboot xenix