August 1992

AT&T UNIX DRIVER PROGRAMS

for

D-Link DE-400 EISA Bus Master Ethernet Card

===============================================

The following files will be used to in this installation:

Files          Contents

-----------    ----------------------------------------------------------

de400.o        DE-400 series driver program

dlk4

strcf

Installation and configuration consist of the following steps:

1.  Login as the root user.

2.  Installing TCP/IP software first.

3.  If the install diskette is DOS format

Copying D-Link driver through the doscp utility

#mkdir /usr/lib/tcprt/dlk

#doscp a:/at\&tunix/de400.o  /usr/lib/tcprt/dlk/de400.o

#doscp a:/at\&tunix/dlk4  /usr/lib/tcprt/dlk/dlk4

#doscp a:/at\&tunix/strcf  /usr/lib/tcprt/dlk/strcf

#doscp a:/at\&tunix/readme.doc  /usr/lib/tcprt/dlk/readme.doc

#chmod +x /usr/lib/tcprt/dlk/dlk4

4.  Installing D-Link Ethernet driver

#cd /usr/lib/tcprt/dlk

#./dlk4

Do you wish to install or delete the dlk driver?  type i to install

A display of information about driver.            type y to continue

Select interrupt vector number                    value

Select solt number                                value

Do you want relink the kernel?                    type y to relink

5.  Reboot unix

#cd /

#shutdown -y -g0 -i6