Software Change/Update Notification

| ID model-year-serial |  |  |  |
| --- | --- | --- | --- |
| .Software/Driver Name |  | NDIS345 |  |
| # .Product Name |  | AN983 |  |
| .Modified From(Which Version) |  | V2.17.10.25.2001 |  |
| .Release File Number |  | two |  |
| # Release Filename | # Version | # Size | # Date |
| FastNIC.SYS | 2.18.0416.2002 | 37k | 04/16/2002 |
| Issue: 2.18.0416.2002 1.. XP Only uses ScatterGatherDMA. 2. Set D0 before getting any information from CSR. 3. Support OID_PNP_WAKE_UP_PATTERN_LIST for WHQL QFE5. 4. Support OID 64 bit for WHQL. 5.Coherence of link status in OID_GEN_MEDIA_CONNECT_STATUS and disMIndicateStatus , while driver just finishes initialization. 6.Changing management of double-copy buffers. |  |  |  |

Approved by: